高分辨率三维工业图像采集设备技术参数

- 千兆网口图像采集器2台

A/D Converter: 14-bit (10- and 12-bit GS3-PGE-23S6)

Video Data Output: 8, 12, 16 and 24-bit digital data

Image Data Formats: Mono8, Mono12, Mono16 (all models) RGB

Partial Image Modes: Pixel binning and region of interest (ROI) modes

Image Processing: Gamma, lookup table, hue, saturation, and sharpness

Shutter: Global shutter;

Gain: Automatic/manual/one-push modes

Gamma: 0.50 to 4.00, programmable lookup table

White Balance: Automatic/manual/one-push modes

High Dynamic Range: Cycle 4 gain and exposure presets

Color Processing: On-camera in YUV or RGB format, or on-PC in Raw format

Transfer Rates: 10/100/1000 Mbit/s

External Trigger Modes: Standard, bulb, low smear (CCD models)

Image Buffer: 128 MB frame buffer

Flash Memory: 2 MB non-volatile memory

Dimensions: 44 mm x 29 mm x 58 mm excluding lens holder

Lens Mount: C-mount

Operating System: Windows, Linux (32- or 64-bit)

- 12mm高清定焦镜头2个

IMAGE FORMAT: 1-inch

FOCAL LENGTH: 12mm

MAXIMUM APERTURE: F1:1.6

APERTURE RANGE: F1.6-22

M.O.D.:0.2mm

ANGLE OF VIEW: HORIZONTAL(55°)VERTICAL(40°)

MOUNT：C-mount

- 采集器支架等附属设备一套

三脚架、固定设备等（根据设备尺寸定制）